Assignment module 2

Section 1: Multiple Choice

1. Which of the following precautions should be taken before working on computer hardware?

a) Ensure the computer is plugged in to prevent electrostatic discharge.

b) Wear an anti-static wrist strap to prevent damage from electrostatic discharge.

c) Work on carpeted surfaces to prevent slipping.

d) Use magnetic tools to handle components more easily

Ans b) Wear an anti-static wrist strap to prevent damage from electrostatic discharge.

Explanation: Before working on computer hardware, it’s important to prevent electricity discharge, which can damage sensitive components like RAM, CPU, and motherboard chips. Wearing an anti-static wrist strap safely grounds you and discharges any static electricity.

2. What is the purpose of thermal paste during CPU installation?

a) To insulate the CPU from heat.

b) To provide mechanical support for the CPU.

c) To improve thermal conductivity between the CPU and the heat sink.

d) To prevent the CPU from overheating.

Ans c) To improve thermal conductivity between the CPU and the heat sink.

Explanation: Thermal paste fills in the tiny air gaps between CPU and Heat sink, which helps heat move from the hot CPU to the cooler heat sink more efficiently.

3. Which tool is used to measure the output voltage of a power supply unit (PSU)?

a) Multimeter

b) Screwdriver

c) Pliers

d) Hex key

Ans a) Multimeter

Explanation: A multimeter is used to measure electrical values. When testing a power supply unit (PSU), a multimeter can check whether the output voltages on the different connectors are correct and stable.

4. Which component is responsible for storing BIOS settings, such as date and time, even when the computer is powered off?

a) CMOS battery

b) CPU

c) RAM

d) Hard drive

Ans a) CMOS battery

Explanation: The CMOS battery provides power to the chip, which stores important system settings such as the BIOS configuration, date, and time even when the computer is turned off or unplugged.

Section 2: True or False

5. True or False: When installing a new hard drive, it is essential to format it before use

Ans True
Explanation: A new hard drive must be formatted before use so the operating system can organize and store data on it properly. Formatting creates a file system (like NTFS or FAT) that allows the drive to function.

6. True or False: A POST (Power-On Self-Test) error indicates a problem with the CPU.

Ans False
Explanation: A POST error doesn’t always mean a CPU problem — it can indicate issues with RAM, graphics card, motherboard, or other hardware components. It’s a general hardware diagnostic test run by the BIOS during startup.

7. True or False: It is safe to remove a USB flash drive from a computer without ejecting it first.

Ans False
Explanation: It is not safe to remove a USB flash drive without ejecting it first, because data might still be transferring. Failing to eject properly can lead to data corruption or loss.

Section 3: Short Answer

8. Describe the steps involved in installing a new graphics card in a desktop computer.

Ans

- Power Down: Completely shut down the computer and unplug it from the wall.

- Open the Case: Remove the side panel of the computer case to get to the internal components.

- Find the Slot: Locate the correct slot on the motherboard, which is usually the longest one, called a PCI-Express (PCIe) x16 slot.

- Prepare the Slot: Remove the small metal bracket on the back of the computer case that lines up with the PCIe slot.

- Insert the Card: Carefully align the graphics card with the slot and press down firmly until it clicks into place.

- Secure the Card: Use a screw to fasten the card's metal bracket to the computer case.

- Connect Power: If the graphics card requires extra power, connect the necessary power cables from the power supply unit (PSU).

- Close Up: Put the side panel back on the case, plug the computer back in, and connect your monitor cable to the new graphics card.

9. What is RAID, and what are some common RAID configurations?

Ans

RAID is a technology that combines multiple hard drives into a single unit for improved performance, reliability, or both.

Common RAID Configurations:

- RAID 0 (Striping):

  - Data is split across two or more drives.

  - Increases performance.

  - No data protection — if one drive fails, all data is lost.

- RAID 1 (Mirroring):

  - Data is duplicated on two drives.

  - High reliability — data is safe if one drive fails.

  - Uses double the storage space.

- RAID 5 (Striping with Parity):

  - Data and parity (error-checking info) are spread across three or more drives.

  - Good balance of speed, storage efficiency, and fault tolerance.

  - Slower write performance.

- RAID 10 (1+0):

  - Combines mirroring and striping using at least four drives.

  - High speed and data protection.

  - Expensive — requires more drives.

Section 4: Practical Application

10. Demonstrate how to replace a CPU fan in a desktop computer.

Ans

Replacing a CPU fan (which is usually attached to a heat sink) involves these steps:

  - First, power off the PC, unplug all cables, and open the computer case.

  - Locate the CPU fan on the motherboard. It's usually a large fan attached to a metal block (the heat sink) sitting on top of the CPU.

  - Carefully unplug the fan's power cable from the motherboard. The plug is usually labeled "CPU_FAN".

  - Unlock the mechanism holding the heat sink and fan assembly in place. This could be push-pins that you twist and pull up, or levers that you flip.

  - Gently twist and lift the entire cooler off the CPU. The old thermal paste might make it a little sticky.

  - Use a lint-free cloth and some isopropyl alcohol to clean the old, dried thermal paste off the top of the CPU.

  - Apply a small, pea-sized dot of new thermal paste onto the center of the clean CPU.

  - Place the new fan and heat sink on top of the CPU, making sure it sits flat.

  - Secure the new cooler using its mounting mechanism. Make sure it's on tight and doesn't wobble.

  - Plug the new fan's power cable into the "CPU_FAN" header on the motherboard

Section 5: Essay

- Discuss the importance of regular maintenance for computer hardware and provide examples of maintenance tasks.

Regular maintenance of computer hardware is essential for keeping a computer running smoothly, extending its lifespan, and preventing unexpected failures. Just like a car needs regular oil changes and check-ups, a computer needs care to perform at its best. Neglecting maintenance can lead to overheating, slow performance, and costly repairs down the road.

One of the most important maintenance tasks is keeping the computer clean. Over time, dust and hair build up inside the computer case, clogging fans and covering components. This layer of dust acts like a blanket, trapping heat. Heat is the enemy of electronics; it can cause components to slow down to protect themselves and can eventually lead to permanent damage. A simple task like using compressed air to blow dust out of fans, vents, and heat sinks every few months can significantly improve cooling and prevent overheating.

Another key area of maintenance is ensuring good airflow and secure connections. This can involve organizing the cables inside the case so they don't block fans. It's also a good practice to occasionally check that all internal components, like RAM sticks and graphics cards, are seated properly in their slots, and that all power and data cables are plugged in firmly. Loose connections can cause random crashes or prevent the computer from starting up at all.

In conclusion, taking a little time for regular hardware maintenance is a wise investment. Simple tasks like cleaning out dust, managing cables, and checking connections help maintain good performance, prevent components from failing due to heat, and ensure the computer remains reliable for years to come. It’s a proactive approach that saves both money and frustration in the long term.